CPE301 – SPRING 2019

# Design Assignment DA3B

Student Name: Saul Alejandro Mendoza Guzman

Student #: 2000540481

Student Email:  mendos1@unlv.nevada.edu

Primary Github address: https://github.com/mendos1

Directory: subnission_da/DA3B

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7, ATmega328p Xplained mini, FTDI chip, breadboard, two wires, two usb cables, LM34, LIGHTER.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL //16MHz

#define BAUD 9600 //Baud Rate

#define MYUBRR F_CPU/16/BAUD-1 //calculate Baud

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

void READ_ADC(void);  //Read ADC

void ADC_INIT(void);  //initialize ADC

void USART_INIT( unsigned int ubrr ); //initialize USART

void USART_TX_STRING(char *data); //Print String USART

volatile unsigned int ADC_TEMP;

char outs[20]; //array

int main(void) {

ADC_INIT(); // Initialize the ADC

USART_INIT(MYUBRR); // Initialize the USART

USART_TX_STRING("Connected!\r\n"); // connection with USART

_delay_ms(125); // wait a bit

sei(); //interrupt

while(1)

{

}

}

void USART_TX_STRING( char *data ) {

while ((*data != '\0')) {

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

/* INIT USART (RS-232) */

void USART_INIT( unsigned int ubrr ) {

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0); // Enable receiver, transmitter & RX interrupt

UCSR0C = (3 << UCSZ00); //asynchronous 8 N 1

}

void ADC_INIT(void)

{

ADMUX = (0<<REFS1)| // Reference Selection Bits

(1<<REFS0)| // AVcc - external cap at AREF

(0<<ADLAR)| // ADC Left Adjust Result

(1<<MUX2)| // ANalog Channel Selection Bits

(0<<MUX1)| //

(1<<MUX0);

ADCSRA = (1<<ADEN)| // ADC ENable

(0<<ADSC)| // ADC Start Conversion

(0<<ADATE)| // ADC Auto Trigger Enable

(0<<ADIF)| // ADC Interrupt Flag

(0<<ADIE)| // ADC Interrupt Enable

(1<<ADPS2)| // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

// Timer/Counter1 Interrupt Mask Register

TIMSK1 |= (1<<TOIE1); // enable overflow interrupt

TCCR1B |= (1<<CS12)|(1<<CS10); // clock

TCNT1 = 49911; //((16MHz/1024)*1)-1 = 15624

}

}

ISR(TIMER1_OVF_vect) //timer overflow interrupt to delay for 1 second

{

READ_ADC();//read ADC

snprintf(outs,sizeof(outs),"%3d F \r\n", ADC_TEMP);// print it

USART_TX_STRING(outs);

TCNT1 = 49911; //reset tcnt

}

void READ_ADC(void) {

unsigned char i =4;

ADC_TEMP = 0; //initialize

while (i--) {

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

ADC_TEMP+= ADC;

_delay_ms(50);

}

ADC_TEMP = ADC_TEMP/8 ; // Average

}

- schematic

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/watch?v=xRrjuo9wgkI

- GITHUB LINK OF THIS DA

https://github.com/mendos1/subnission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT